# Homework 10: AE302, High-Speed Aerodynamics

### April 23, 2008

### Due: April 30, 2008

Problem 1: 11.1, 11.2

Problem 2:

A two-dimensional airfoil with a triangular shape is placed in a supersonic flow. The top side of the profile is parallel with the free-stream velocity direction:

3 degrees

c

t

M∞=2.1

δ2

δ1

Given:

- M∞=2.1.

- Wedge angle = 3 degrees.

- The pressure coefficient on the backside of the profile is given by cp,∞=-0.0706. (Two contact discontinuities form at the back of the wedge).

- Determine the lift and drag coefficient on the airfoil using shock-expansion theory.

- Determine the deflection angles δ1 and δ2 of the flow on the back of the airfoil.