# Homework 7: AE430, Aircraft Propulsion Systems

### November 15, 2007

### Due: November 27, 2007, beginning of class

Problem 1:   6.2

Problem 2:   6.4

Problem 3:   A jet operates at an altitude of 10,000 m (Ta=223oK, pa=0.26 atm, γ=1.4) at a Mach no. of 1.7. The engine mass flow is limited by a throat are of At=500cm2. The external diffusion is based on an oblique shock and on a normal shock, as described in the figure.

Calculate:

- Stagnation pressure recovery, p0,2/p0,a?

- At what Mach no. does the oblique shock become detached?

- What is the distance x,  from the cone tip to the outer inlet lip, for the condition described in the figure?

What is the best turning angle θ in terms of highest pressure ratio, p0,2/p0,a?

15o

0.4m

Ma=1.7

x