# Homework 8: AE430, Aircraft Propulsion Systems

### November 29, 2007

### Due: December 4, 2007, beginning of class

Problem 1.

A single stage compressor rotates at 4000 RPM. The incoming flow speed is cz=40 m/s, the rotor average radius is rm = 0.25 m, and the rotor blade consists of a symmetric airfoil shape oriented at 60 deg (β2=600). Also ηst=0.85, γ=1.4, cp=1.004 kJ/kgK, and T0,1=300oK.

- Draw the velocity diagram for the rotor

- Calculate rotor angle β1

- Calculate stagnation pressure ratio P02/P01

- Calculate torque and power requirements

Problem 2

The axial velocity across the second stage of an axial compressor is cz= 120 m/sec, the average radius is rm = 0.4 m, and the stagnation temperature ahead of the rotor is To = 300oK. If the compressor rotates at 5000 RPM, the angle α1= 25 deg, and β2 = 30 deg, calculate:

- The angles α2 and β1.

- The torque required to drive the rotor.

- The stagnation pressure rise p03/p01 across this stage. (ηst= 0.98, cp= 1.004 kJ/kgK, γ = 1.4).

l